# OPos Driver Manual

## Driver Install

- 1.1 Run “RTOPos.exe: to install the printer driver

- 1.2 Select installation directory. Click ”Next”.

- 1.3 Do you want to create a desktop shortcut? If not, click“Next”.

- 1.4 Click ”Install”

- 1.5 Click ”Finish”

## 2.Configure the OPOS printer

- Please connect the printer before configuration.Run RTOposConfigTool.exe。as shown in the figure below.

### 2.1 Create logical driver name

Enter the printer name in the LDN，Click “New”. After it will be displayed in device name.

### 2.2.Communication interface configuration

- The available interfaces are COM, ENet, LPT and USB.as shown in the figure below.

#### 2.2.1 Configure COM Port

- Select the interface type as com. Click port configure to set it.

- Select the corresponding COM port and click “OK”,as shown in the figure below.

- The information bar will display the corresponding configuration information. Click Save to complete the configuration.

#### 2.2.2 Configure ENet Port

- Select the interface type as Enet. Click port configure to set it.as shown in the figure below.

- Enter the IP address of the printer and click “OK”

- The information bar will display the corresponding configuration information. Click Save to complete the configuration.

#### 2.2.3 Configure LPT Port

- Select the interface type as LPT. Click port configure to set it.as shown in the figure below.

- Select the corresponding LPT port and click "OK"

- The information bar will display the corresponding configuration information. Click Save to complete the configuration.

#### 2.2.4 Configure USB Port

- Select the interface type as USB. Click port configure to set it.as shown in the figure below.

- Select the corresponding USB port and click "OK"

- The information bar will display the corresponding configuration information. Click Save to complete the configuration.

#### 2.2.5 Delete OPOS Printer Configuration

- Select the device name to be deleted in the device name list and click “Delete”.

## 3.Configure Cash Drawer

- The printer needs to be configured before configuring the cash drawer.

### 3.1 Create logical driver name

Enter the printer name in the LDN，Click “New”. After it will be displayed in device name.

### 3.2 Save the cash drawer configuration

- Select the corresponding print,Set the Connector pin and the time of pulse on and off ,Finally, click “Save”.as shown in the figure below.

### 3.3 Delete Cash Drawer Configuration

- Select the device name to be deleted in the device name list and click “Delete”.

## 4.OPOS Test

- Open the OPOS driver installation directory, Run “RTOposTest.exe”.as shown in the figure below.

### 4.1Printer Test

- You must create printer logical driver name before running “RTOposTest.exe”. After successful creation , It will be displayed in LAN list..as shown in the figure below.

- Then click Open -- > claim -- > deviceenable. The execution results can be viewed in the output result. After successful execution, the function button will become available, as shown in the following figure.

### 4.2 Drawer Test

- You must create drawer logical driver name before running “RTOposTest.exe”. After successful creation , It will be displayed in LAN list.as shown in the figure below.

- Then click Open -- > Claim -- > DeviceEnable. After successful execution, the function button will become available, as shown in the figure below.

## Uninstall RTOPOS

- Open Control Panel -->Click Uninstall a Programs-->Select RTOPOS

- Double-click RTOPOS,Click “Yes”,as shown in the figure below.